I-MAP: Logbook (AI Usage)

TSMK

Universiteit van Antwerpen

Toegepaste Ingenieurswetenschappen (Elektronica-ICT)

LLM request

ChatGPT was used for this prompt.

I have a car module that i want to make. It uses a

- Gens ace 4S Lipo Battery 5000mAh 14.8V 50C 4S Bashing RC Battery as its power source

- Corally Kuron 825 4-Pole 2050KV Brushless Sensorless Electric Motor (and its driver) to drive the wheels

- A Teensy 4.1

- A NVIDIA JETSON

- Many 3V3 and 5V pheripherals

My calculations state that (except for the motor) everything would use around 2.7A of current. To be sure that it isn't exceeded I plan to provide a 4A 5V source to these modules (except for the motor) using the TMDC 20-2411.

This would allow me to seperate the battery and motor on a seperate PCB.

Does this provide any benifit (regarding to noise on the ground plane for example)?

LLM resopnse

Short answer: yes, it can provide real benefits, but only if it’s done thoughtfully. Simply splitting things onto another PCB doesn’t magically fix noise—but power-domain separation + proper grounding absolutely helps in systems like yours. Let’s break it down in a practical, no-hand-waving way. 1. Your system is exactly the kind that benefits from separation You have: - A high-current, high-di/dt noise monster → 4S LiPo + sensorless brushless motor + ESC - Sensitive digital + mixed-signal electronics → Jetson Nano, Teensy 4.1, 3.3 V / 5 V peripherals That motor/ESC combo will generate: - Large current spikes - High-frequency switching noise - Ground bounce - EMI that loves to couple into logic rails So you’re right to be thinking about this early 👍 2. What separating the PCBs actually gives you ✅ Real benefits if done right Separating motor power + ESC from logic power helps with: a) Ground noise containment - Motor currents return through their own copper - Logic ground stays quieter - Less chance of: - Teensy resets - Jetson brownouts - USB / UART weirdness - ADC noise b) Cleaner power regulation Using a dedicated DC/DC (TMDC 20-2411) means: - Motor current spikes don’t pull down your 5 V rail - The Jetson sees a much stiffer supply - You avoid “battery sag → logic reset” moments c) EMI control You can: - Keep high dI/dt loops physically small - Route motor phase currents nowhere near logic traces - Add filtering only where it matters This is huge in RC-style builds. 3. Important reality check: PCB separation alone is not enough Here’s the part many people miss 👇 The benefit comes from power-domain separation, not PCB count. You still must: 🔴 Use a single-point (star) ground - Battery negative = common reference - Motor/ESC ground and logic ground meet at one controlled point - Don’t daisy-chain logic ground through the motor PCB Best practice: Battery - ¦ +-- Motor / ESC PCB ¦ +-- DC/DC → Logic PCB 🔴 Avoid shared high-current return paths - No logic signals returning through motor ground copper - No thin traces carrying both logic return + motor noise 4. About the TMDC 20-2411 choice This is actually a solid approach 👍 Benefits: - Isolated or well-filtered DC/DC (depending on config) - Handles your ~2.7 A comfortably with margin - Gives you: - A clean 5 V rail - Room for LDOs down to 3.3 V afterward Strong recommendation: Add post-regulation filtering: - LC or π filter after the DC/DC - Bulk capacitance near Jetson input - Local decoupling per device Jetson Nano in particular is very sensitive to supply dips. 5. Motor controller (ESC) noise deserves special handling Sensorless brushless ESCs are notoriously noisy. Do this even if PCBs are separate: - Keep motor phase wires twisted - Keep battery → ESC loop as short as possible - Add: - Low-ESR bulk caps near ESC - Optional TVS on battery input - Physically distance ESC from Jetson/Teensy if possible Mechanical separation matters just as much as electrical. 6. Summary — does it provide benefit? ✅ Yes, if: - You separate power domains, not just boards - You use star grounding - You prevent shared return paths - You add proper filtering ❌ No, if: - Grounds reconnect all over the place - Logic ground shares motor current paths - The DC/DC output isn’t filtered TL;DR recommendation Your plan is architecturally sound and aligns with how professional robotics and RC-autonomy platforms are built. If you want, next we can: - Sketch a recommended grounding topology - Talk Jetson Nano power best practices - Review signal isolation between Teensy ↔ ESC - Or sanity-check your current calculations under load Just tell me where you want to zoom in 🚗⚡