I-MAP: Logbook (AI Usage)

TSMK

Universiteit van Antwerpen

Toegepaste Ingenieurswetenschappen (Elektronica-ICT)

LLM request

Claude was used for this prompt.

I got this struct for a BNE085 IMU sensor: /** @brief datastruct of IMU  measurement data */ struct movement { float roll; /** @brief roll  defines rolling*/ float pitch; /** @brief pitch defines nose up or  down*/ float yaw; /** @brief yaw defines left right swing*/ float w; /**  @brief real part of quaternionv*/ float i; /** @brief i part*/ float j;  /** @brief j part*/ float k; /** @brief k part*/ }; typedef struct  movement movement_t; But I'm using a PIM448, that provides these values:  float getAccelX() { return _accel.x } float getAccelY() { return  _accel.y } float getAccelZ() { return _accel.z } float getGyroX() {  return _gyro.x } float getGyroY() { return _gyro.y } float getGyroZ() {  return _gyro.z } float getMagX() { return _mag.x } float getMagY() {  return _mag.y } float getMagZ() { return _mag.z } float getTemperature()  { return _temp } float getResultantG() { return _resultantG } How can i  convert them?

LLM resopnse

The PIM448 gives you raw sensor data (accelerometer, gyroscope, magnetometer), while the movement_t struct expects fused orientation data (Euler angles + quaternion). You need a sensor fusion algorithm to bridge them. The Core Problem PIM448 output; movement_t needs. Raw accel XYZ; roll, pitch, yaw. Raw gyro XYZ; w, i, j, k (quaternion). Raw mag XYZ; (used for yaw/heading). Option 1: Mahony Filter (Recommended — lightweight, runs on MCUs) c #include <math.h> // Mahony filter state static float q0 = 1.0f, q1 = 0.0f, q2 = 0.0f, q3 = 0.0f; static float integralFBx = 0.0f, integralFBy = 0.0f, integralFBz = 0.0f; #define SAMPLE_FREQ 100.0f // Hz — match your actual update rate #define TWO_KP 2.0f // Proportional gain #define TWO_KI 0.005f // Integral gain static float invSqrt(float x) { return 1.0f / sqrtf(x); } void mahony_update(float ax, float ay, float az, float gx, float gy, float gz, float mx, float my, float mz) { float recipNorm; float hx, hy, bx, bz; float halfvx, halfvy, halfvz, halfwx, halfwy, halfwz; float halfex, halfey, halfez; // Normalise accelerometer recipNorm = invSqrt(ax*ax + ay*ay + az*az); ax *= recipNorm; ay *= recipNorm; az *= recipNorm; // Normalise magnetometer recipNorm = invSqrt(mx*mx + my*my + mz*mz); mx *= recipNorm; my *= recipNorm; mz *= recipNorm; // Reference direction of Earth's magnetic field hx = 2.0f*(mx*(0.5f - q2*q2 - q3*q3) + my*(q1*q2 - q0*q3) + mz*(q1*q3 + q0*q2)); hy = 2.0f*(mx*(q1*q2 + q0*q3) + my*(0.5f - q1*q1 - q3*q3) + mz*(q2*q3 - q0*q1)); bx = sqrtf(hx*hx + hy*hy); bz = 2.0f*(mx*(q1*q3 - q0*q2) + my*(q2*q3 + q0*q1) + mz*(0.5f - q1*q1 - q2*q2)); // Estimated direction of gravity and magnetic field halfvx = q1*q3 - q0*q2; halfvy = q0*q1 + q2*q3; halfvz = q0*q0 - 0.5f + q3*q3; halfwx = bx*(0.5f - q2*q2 - q3*q3) + bz*(q1*q3 - q0*q2); halfwy = bx*(q1*q2 - q0*q3) + bz*(q0*q1 + q2*q3); halfwz = bx*(q1*q3 + q0*q2) + bz*(0.5f - q1*q1 - q2*q2); // Error is cross product of estimated and measured directions halfex = (ay*halfvz - az*halfvy) + (my*halfwz - mz*halfwy); halfey = (az*halfvx - ax*halfvz) + (mz*halfwx - mx*halfwz); halfez = (ax*halfvy - ay*halfvx) + (mx*halfwy - my*halfwx); // Apply integral feedback integralFBx += TWO_KI * halfex * (1.0f / SAMPLE_FREQ); integralFBy += TWO_KI * halfey * (1.0f / SAMPLE_FREQ); integralFBz += TWO_KI * halfez * (1.0f / SAMPLE_FREQ); gx += integralFBx; gy += integralFBy; gz += integralFBz; // Apply proportional feedback gx += TWO_KP * halfex; gy += TWO_KP * halfey; gz += TWO_KP * halfez; // Integrate rate of change of quaternion float dt = 1.0f / SAMPLE_FREQ; q0 += (-q1*gx - q2*gy - q3*gz) * (0.5f * dt); q1 += ( q0*gx + q2*gz - q3*gy) * (0.5f * dt); q2 += ( q0*gy - q1*gz + q3*gx) * (0.5f * dt); q3 += ( q0*gz + q1*gy - q2*gx) * (0.5f * dt); // Normalise quaternion recipNorm = invSqrt(q0*q0 + q1*q1 + q2*q2 + q3*q3); q0 *= recipNorm; q1 *= recipNorm; q2 *= recipNorm; q3 *= recipNorm; } Option 2: Fill the movement_t struct Call this after mahony_update() each cycle: c #define RAD_TO_DEG 57.29577951f movement_t pim448_to_movement(PIM448 &imu) { // Feed the filter (gyro must be in rad/s — convert if in deg/s) mahony_update( imu.getAccelX(), imu.getAccelY(), imu.getAccelZ(), imu.getGyroX(), imu.getGyroY(), imu.getGyroZ(), // ensure rad/s! imu.getMagX(), imu.getMagY(), imu.getMagZ() ); movement_t m; // Quaternion components m.w = q0; m.i = q1; m.j = q2; m.k = q3; // Euler angles from quaternion m.roll = atan2f(2.0f*(q0*q1 + q2*q3), 1.0f - 2.0f*(q1*q1 + q2*q2)) * RAD_TO_DEG; m.pitch = asinf(2.0f*(q0*q2 - q3*q1)) * RAD_TO_DEG; m.yaw = atan2f(2.0f*(q0*q3 + q1*q2), 1.0f - 2.0f*(q2*q2 + q3*q3)) * RAD_TO_DEG; return m; } Key things to watch out for - Gyro units: Mahony expects rad/s. If your PIM448 driver returns deg/s, multiply by M_PI / 180.0f before passing in. - Sample rate: Set SAMPLE_FREQ to match how often you actually call the function — drift will accumulate if it's wrong. - Magnetometer calibration: Hard-iron distortion will skew your yaw. You'll want to capture min/max on all axes and subtract the offsets. - Gimbal lock: asinf for pitch will have singularities near ±90°. If you need full 3D rotation, work with the quaternion directly and skip Euler angles. You said: Note that the PIM gets gyroscope data in degrees/s. Note that the PIM gets gyroscope data in degrees/s. Does that change anything in the code? 24 Apr Claude responded: Yes, just one change needed — convert the gyro inputs from deg/s to rad/s before passing them into mahonyupdate(). Yes, just one change needed — convert the gyro inputs from deg/s to rad/s before passing them into mahony_update(). Update the call in pim448_to_movement(): c #define DEG_TO_RAD 0.01745329252f // M_PI / 180.0f movement_t pim448_to_movement(PIM448 &imu) { mahony_update( imu.getAccelX(), imu.getAccelY(), imu.getAccelZ(), imu.getGyroX() * DEG_TO_RAD, // convert deg/s → rad/s imu.getGyroY() * DEG_TO_RAD, imu.getGyroZ() * DEG_TO_RAD, imu.getMagX(), imu.getMagY(), imu.getMagZ() ); // ... rest unchanged } The mahony_update() function itself stays exactly as written — no changes there. Everything else (quaternion integration, Euler angle conversion) operates in radians already, so that one multiplication is the only fix needed.