I-MAP: Logbook (AI Usage)

TSMK

Universiteit van Antwerpen

Toegepaste Ingenieurswetenschappen (Elektronica-ICT)

LLM request

Claude was used for this prompt.

i'm using a NEO 6M gps module. It says online that it only works with baudrate 9600, could i possibly increase it?

LLM resopnse

Yes, you can increase the baud rate of the NEO-6M beyond 9600! Here's how it works: How to Change the Baud Rate The NEO-6M's default baud rate is 9600, but it supports higher rates. You change it by sending a UBX configuration command over the serial interface. Supported baud rates: 4800, 9600, 19200, 38400, 57600, 115200, 230400 Method 1: UBX CFG-PRT Command (Recommended) Send this byte sequence at 9600 to switch to 115200: 0xB5 0x62 0x06 0x00 0x14 0x00 ← Header + Class + ID + Length 0x01 ← Port ID (UART1) 0x00 ← Reserved 0x00 0x00 ← TX ready (disabled) 0xD0 0x08 0x00 0x00 ← UART mode (8N1) 0x00 0xC2 0x01 0x00 ← Baud rate = 115200 in little-endian 0x07 0x00 ← Input protocols (UBX+NMEA+RTCM) 0x03 0x00 ← Output protocols (UBX+NMEA) 0x00 0x00 ← Flags 0x00 0x00 ← Reserved 0xBC 0x5E ← Checksum (CK_A, CK_B) After sending this, immediately switch your host's baud rate to 115200 — the module changes instantly. Method 2: u-center Software If you have a USB-to-TTL adapter, use u-blox's free u-center tool: - Connect at 9600 - Go to View → Configuration View → PRT (Ports) - Change the baud rate and click Send - Optionally click CFG → Save config to persist it

LLM request

could i do this via linux, since i only have a debain 13 system?

LLM resopnse

Yes, Linux works great for this! You can send the UBX command directly via the terminal. Step 1 — Find the Arduino's Port bash ls /dev/ttyUSB* /dev/ttyACM* It'll likely show up as /dev/ttyACM0. Step 2 — Upload the Empty Sketch You need the Arduino IDE or arduino-cli to upload the passthrough sketch. The quickest way on Debian: bash sudo apt install arduino-cli arduino-cli core install arduino:avr # Save the empty sketch mkdir -p ~/gps_config && cat > ~/gps_config/empty/empty.ino << 'EOF' void setup() {} void loop() {} EOF # Upload it arduino-cli compile --fqbn arduino:avr:uno ~/gps_config/empty arduino-cli upload -p /dev/ttyACM0 --fqbn arduino:avr:uno ~/gps_config/empty Step 3 — Send the UBX Command via Python Once the empty sketch is uploaded and the NEO-6M is wired to pins 0/1: bash sudo apt install python3-serial Then create and run this script: python # ~/gps_config/set_baud.py import serial import time PORT = "/dev/ttyACM0" # UBX CFG-PRT: switch UART1 to 115200 cfg_prt = bytes([ 0xB5, 0x62, 0x06, 0x00, 0x14, 0x00, 0x01, 0x00, 0x00, 0x00, 0xD0, 0x08, 0x00, 0x00, 0x00, 0xC2, 0x01, 0x00, # 115200 0x07, 0x00, 0x03, 0x00, 0x00, 0x00, 0x00, 0x00, 0xBC, 0x5E ]) # UBX CFG-CFG: save to flash cfg_save = bytes([ 0xB5, 0x62, 0x06, 0x09, 0x0D, 0x00, 0x00, 0x00, 0x00, 0x00, 0xFF, 0xFF, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x17, 0x31, 0xBF ]) print("Connecting at 9600...") with serial.Serial(PORT, 9600, timeout=1) as ser: time.sleep(2) # let port settle print("Sending CFG-PRT (switch to 115200)...") ser.write(cfg_prt) time.sleep(0.1) print("Reopening at 115200...") with serial.Serial(PORT, 115200, timeout=1) as ser: time.sleep(0.5) print("Sending CFG-CFG (save to flash)...") ser.write(cfg_save) time.sleep(0.2) # Verify it's working by reading NMEA output print("\nReading GPS output (Ctrl+C to stop):") while True: line = ser.readline() if line: print(line.decode('ascii', errors='replace').strip()) bash python3 ~/gps_config/set_baud.py If you see NMEA sentences like $GPRMC,... scrolling by at the end, it worked!

LLM request

What byte arrays do i need to send to increase the baud rate and refresh speed?
Also are there data points that i can remove?

LLM resopnse

/** @brief UBX CFG-PRT command: Switch baud rate to 115200. */ const byte UBX_BAUD[28] = { 0xB5, 0x62, 0x06, 0x00, 0x14, 0x00, 0x01, 0x00, 0x00, 0x00, 0xD0, 0x08, 0x00, 0x00, 0x00, 0xC2, 0x01, 0x00, // 115200 baud 0x07, 0x00, 0x03, 0x00, 0x00, 0x00, 0x00, 0x00, 0xC0, 0x7E }; /** @brief UBX CFG-RATE command: Set update rate to 10Hz. */ const byte UBX_10HZ[14] = {0xB5,0x62,0x06,0x08,0x06,0x00,0x64,0x00,0x01,0x00,0x01,0x00,0x7A,0x12}; /** @brief UBX CFG-MSG command: Disable GLL sentence. */ const byte DIS_GLL[16] = {0xB5,0x62,0x06,0x01,0x08,0x00,0xF0,0x01,0x00,0x00,0x00,0x00,0x00,0x00,0x00,0x2A}; /** @brief UBX CFG-MSG command: Disable GSA sentence. */ const byte DIS_GSA[16] = {0xB5,0x62,0x06,0x01,0x08,0x00,0xF0,0x02,0x00,0x00,0x00,0x00,0x00,0x00,0x01,0x31}; /** @brief UBX CFG-MSG command: Disable GSV sentence. */ const byte DIS_GSV[16] = {0xB5,0x62,0x06,0x01,0x08,0x00,0xF0,0x03,0x00,0x00,0x00,0x00,0x00,0x00,0x02,0x38}; /** @brief UBX CFG-MSG command: Disable VTG sentence. */ const byte DIS_VTG[16] = {0xB5,0x62,0x06,0x01,0x08,0x00,0xF0,0x05,0x00,0x00,0x00,0x00,0x00,0x00,0x04,0x46};